# METHOD OF STATEMENT

Date: 27th July 2023

Description of work

To unload 11 cases of 1 unit machine from 4 x 40ft HQ container onto lorry chassis, transport lorry into inside factory, using customer overhead crane to unload and position to designated location

Machine Spec

Kuraki CNC Horizontal Boring & Milling / KBT-13EA

Refer to packing list attached

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

03-04/08/23

Time of Operation

09:45 am

Location of Operation

#### No 3 Joo Koon Circle

AME International Pte Ltd

Equipment Use

1-2 x lorry loader

Customer overhead crane

1 x forklift

Wire/webbing sling

Shackle

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport of lifting machinery into side

- Deploy lorry loader and park at designated unloading area  (back to back with container)

- Using lorry crane to pull out machine cases from container

- Load case onto lorry chassis

- Transport lorry into factory production area

- Using customer overhead crane to unload case from lorry chassis

- Rigger to insert sling to overhead crane hook and secured to machine rigging point

- Hoist up and detach machine from base

- Shift and position machine at the designated location

- Unload balance item and put aside

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175