# METHOD OF STATEMENT

Date: 26th July 2018

Description of work

To unload 5 pallet of 1 unit machine from 2 x 40ft HQ container onto lorry chassis, transport into customer production area, hoist down and position to designated location

Machine Spec

1 unit used Ikegai Horizontal Boring Machine

Pallet 1: Net Wt: 9, 800 kgs

Pallet 2: Net Wt: 5, 130 kgs

Pallet 3: Net Wt: 4, 120 kgs

Pallet 4: Net Wt: 5, 250 kgs

Pallet 5: Net Wt: 4, 780 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

26/07/2018   -    10:00 am

Location of Operation

#### No 3 Joo Koon Circle

AME International Pte Ltd

Equipment Use

1 x 50 ton lorry crane

Wire sling

Shackle

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

# Job Scope

- Lorry crane to park at designated location (back to back with container)

- Pull out machine base from container and load onto lorry chassis

- Transport lorry with machine into customer production area

- Hoist and detach machine from base

- Swing crane and lower down machine to near ground level

- Position and unload to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175