# METHOD OF STATEMENT

Date: 18th May 2017

Description of work

To unload 1 unit machine from 1 x 40ft HQ container onto lorry chassis, transport into customer production area, hoist down and position to designated location

Machine Spec

1 unit HM-300 High Speed Precision Hydraulic Rubber Molding Machine

Dim: 1700 x 2425 x 3110 mm

Wt; 14, 435 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Est: 24/05/2017   -    10:00 am

Location of Operation

#### No 10 Tuas South St 5

CDI Multiseals Pte Ltd

Equipment Use

1 x 50 ton lorry crane

Wire sling

Shackle

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc... safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

# Job Scope

- Lorry crane to park at designated location (back to back with container)

- Pull out machine from container and load onto lorry chassis

- Transport lorry with machine into customer production area

- Hoist down machine from lorry chassis

- Lower down machine to near ground level,

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175