# METHOD OF STATEMENT

Date: 09th May 2017

Description of work

To hoist up 1 unit machine to level 3 mezzanine floor, load onto pallet truck, shift and position to designated location

Machine Spec

1 x Horizontal Bandsaw machine, Model H-330A/GA

Dim: 220 x 227 x 170 cm

Wt: 1630 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

12/05/2017   -   11:00 am

Location of Operation

#### No 86 International Rd

Gliderol Door (S) Pte Ltd

Equipment Use

1 x 45 ton lorry crane

Wire/webbing sling

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc... safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

# Job Scope

- Transport lorry crane with machine into customer workshop

- Insert wire/webbing sling onto machine hoisting point

- Lorry crane to hoist up machine to level 3rd mezzanine floor

- Lower down machine slowly to near ground level

- Insert pallet truck to machine base

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175