# METHOD OF STATEMENT

Date: 07th June 2018

Description of work

Transport lorry crane into production area, hoist and shift out 1 unit machine table, to tilt up table for engineer to do maintenance, after maintenance, install back onto machine

Machine Spec

1 unit Gun-drill table

Dim: 2m x 2m

Est Wt: 4, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

11-13/06/2018    -    10:00 am

Location of Operation

#### 11 Tuas South Ave 12

Halliburton Completions MFG Pte Ltd

Equipment Use

1 x lorry crane

Wire/webbing sling

Shackle

Canvas/metal sheet (for epoxy floor protection)

Obstacles

Clear obstruction in way of working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport lorry crane into customer production area

- To hoist up table from machine body

- Swing crane to open space

- Tilt up table with support of chain block or customer overhead crane for engineer to do maintenance

- After maintenance is done, to install back table onto machine

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175