# METHOD OF STATEMENT

Date: 30th June 2025

Description of work

Reverse back of lorry into inside factory, using overhead crane to hoist up equipment and load onto lorry chassis, transfer to another bay, use overhead crane to unload equipment, shift and position to location

Spec

1 unit Equipment

Wt: 1, 800 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/07/25

Time of Operation

09:30 am

Location of Operation

149 Gul Circle

FMC Technologies Singapore Pte Ltd

Equipment Use

1 x 633 lorry crane

Webbing sling

Shackle

Rollers/pallet truck

Crowbar/woodblock

Obstacles

Clear obstruction in way of working area and route to structure position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Lift up equipment using crowbar

- Insert pallet truck under equipment bed

- Transport equipment to near overhead crane

- Riggers to insert sling to crane hook

- Secured sling to rigging point of equipment

- Hoist up equipment

- Reverse back of lorry into factory and load up equipment onto lorry chassis

- Transfer to another bay

- Unload equipment from lorry chassis using overhead crane

- Transport equipment to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175