# METHOD OF STATEMENT

Date: 03rd Aug 2021

Description of work

To assist Mazak engineer in installation of machine parts

Machine parts

Dim: 472 x 257 x 263 cm / Wt: 5, 940 kg

Dim: 422 x 237 x 188 cm / Wt: 3, 580 kg

Dim: 850 x 221 x 148 cm / Wt: 1, 930 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10/08/21

Time of Operation

09:45 am

Location of Operation

#### 18 Joo Koon Circle

Yamazaki Mazak (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Wire/webbing sling

Shackle

Obstacles

Clear obstruction in way of working area and route of transportation

Barricade operation area to prevent persons who are not involved enter unintentionally

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport lorry loader into inside factory premise

- Insert required sling to crane hook and secure onto hoisting point of machine part to be hoisted

- Hoist up machine part slowly to check on balancing

- When well balance, swing crane to the required point for installation

- Assist engineer in installing the part

- Repeat process until all parts are assemble

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175