# METHOD OF STATEMENT

Date: 06th June 2025

Description of work

Using of factory overhead and lorry crane to hoist and upright column, after upright, overhead crane to position to designated location

Spec

2 set machine column

Wt: 4, 500 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

2 x job:

13/06/25

16/06/25

Time of Operation

09:30 am

Location of Operation

149 Gul Circle

FMC Technologies Singapore Pte Ltd

Equipment Use

1 x 633 lorry crane

Factory 20ton overhead crane

Wire sling

Webbing sling

Shackle

Obstacles

Clear obstruction in way of working area and route to structure position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport lorry crane into inside production area

- Deploy lorry loader at designated hoisting location

- Set up crane with outrigger fully extended and resting on timber mats as base plate

- Riggers to insert sling to overhead crane hook

- Secured sling to rigging point of column

- Overhead crane to lift up column

- Using lorry crane as support to upright column

- After upright, overhead crane to position column at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris or wood waste will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175