# METHOD OF STATEMENT

Date: 17th Mar 2021

Description of work

To reverse lorry in front of tenant premise, unload 1 unit machine from lorry chassis, shift and position into customer premise at ground

Machine Spec

1 unit YCM NXV 1020A CNC

Dim: 340 x 227 x 257 cm

Wt: 5, 700 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

27/03/2021 (Saturday)

Time of Operation

13:00 pm

Location of Operation

#### Blk 5012 A.M.K Ave 5 #01-04

Akribis Systems Pte Ltd

Equipment Use

1-2 x 633 lorry loader

1 set wire sling

Rollers/pallet truck

Metal plate

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones/red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting operation are supervised by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry loader in front of tenant door entrance

- Flush up lorry to level with loading bay

- Lorry loader to hoist up machine from lorry chassis

- Insert rollers/pallet truck under machine base

- Shift out machine from lorry chassis into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Remark:

To dismantle sun breaker and make good upon job completion

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175