# METHOD OF STATEMENT

Date: 12th Aug 2020

Description of work

To unload 1 unit machine from lorry chassis, shift and position into customer premise at ground

Machine Spec

1 unit Cutex 160B machine

Dim: 384 x 218 x 216 cm

Wt: 4, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

15/08/2020 (Saturday)

Duration of Operation

08:30 am – 10:30 am

Location of Operation

#### Blk 5012 A.M.K Ave 5 #01-10

LHY Engineering (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones/red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

# JOB SCOPE

- Reverse lorry loader in front of tenant door entrance

- Flush up lorry to level with loading bay

- Jack up and insert rollers under machine base

- Shift out machine from lorry chassis into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175