# METHOD OF STATEMENT

Date: 08th Jan 2021

Description of work

To shift out machineries from tenant premise at ground floor to loading platform, hoist up and load onto low-bed/lorry chassis

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12-18/01/2021

Time of Operation

Location of Operation

#### Blk 5004 A.M.K Ave 5 #01-10

GS Precision Pte Ltd

Equipment Use

1 x static 633 lorry loader (for hoisting machine from loading platform to movable low-bed/lorry

1 x static low-bed (act as platform for machine loading)

1 x low-bed

2-3 lorry

Rollers/pallet truck

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones/red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gears with M.O.M certificate

Lifting operation are supervised by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry loader in front of tenant door entrance

- Flush up lorry to level with loading bay

- Jack up and insert rollers under machine base

- Shift out machine from lorry chassis into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175