# METHOD OF STATEMENT

Date: 16th Aug 2021

Description of Work

To unload machine and parts from trailer, lowbed using forklift and lorry loader, uncrate, shift and position to the designated location at ground floor

Machine Spec

Vacuum Heat-Treatment Line

Refer to packing list

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

18/08/21 – 30/10/21

Time of Operation

09:45 am

Location of Operation

#### 11 Bulim Walk

Shimano (S) Pte Ltd

Equipment Use

633 lorry loader

1 x 100ton lorry loader

1 x 16ton forklift

1 x 4.5ton forklift

Trailer/lowbed

Wire/webbing sling

Shackle

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator and forklift operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy 100 ton lorry loader at designated loading area

- Reverse lowbed to near crane

- 100 ton lorry loader to hoist down case to the ground

- Uncrate case

- Attached lifting gears to crane hook and secured to load to be lifted

- Hoist up and detach machine from base

- Load back machine onto lowbed chassis

- Transport lorry and lowbed into inside factory premise

- Attached lifting gears onto overhead crane hook and secured to load to be lifted

- Hoist down machine from lowbed and load to near ground level

- Swing crane and lower down machine at the designated location

- To assist customer engineer to install machine parts

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175