# METHOD OF STATEMENT

Date: 26th Aug 2025

Description of work

To shift out 5 units machine from production area to loading bay, hoist up onto low-bed/lorry chassis and deliver to new location

Machine Spec

1 unit PSG C-60100 AHR Surface Grinder / Wt: 6, 063 kg

1 unit Youji YV800 ATC-3 / Wt: 20, 500 kg

1 unit Hwacheon VT650R / Wt: 10, 100 kg

1 unit Hwacheon L1-1500 / Wt: 16, 300 kg

1 unit VL1600 / Wt: 8, 740 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

17-18/09/25 – move out (PSG C-60100, Youji YV800, Hwacheon VT650R)

23-24/09/25 – move out (Hwacheon L1-1500, VL1600)

Time of Operation

10:00 am

Location of Operation

#### 2 Seletar Aerospace Link

Avimac Pte Ltd

Equipment Use

1 x 110 ton lorry crane

2 x 633 lorry loader

1 x low-bed

Rollers/pallet truck

Wire/webbing sling

Hydraulic/hand jack

Woodblock

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated loading area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Shift out machine from production to loading area

- Rigger to insert sling to crane hook

- Secure sling to rigging point of machine

- Lorry crane to hoist up machine and load onto low-bed/lorry chassis

- Secured machine with belt prior to moving out of customer premise

- Transport and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175