# METHOD OF STATEMENT

Date: 09th Dec 2025

Description of work

To shift out 1 unit machine and accessories from production at ground floor to loading bay, hoist up onto low loader and transport to another location

Machine Spec

1 x DMG Lasertec 4300 machine

Dim: 550 x 340 x 310 cm

Wt: 31, 720 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

19/12/25 – jack up machine to standby for next day moving

20/12/25 – shift out machine from production and hoist up onto low-bed

Time of Operation

09:30 am

Location of Operation

#### 3 Cleantech Loop, Cleantech 2A

ARTC

Equipment Use

1 x 100ton mobile crane

1 x 633 lorry loader

1 x low bed

Rollers/pallet truck

Wire/webbing sling

Shackle

Hydraulic jack

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

P.P.E Required

Safety shoe

Safety helmet

Glove

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deploy mobile crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Shift out machine from production to loading area

- Rigger to insert sling to crane hook

- Secure sling to rigging point of machine

- Mobile crane to hoist up machine

- Reverse low bed to near crane

- Unload machine onto low loader chassis

- Release all sling from crane hook

- Secure machine with chains

- Transport out and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175