# METHOD OF STATEMENT

Date: 09th July 2025

Description of work

To shift out 3 unit machine from production area to loading bay, hoist up onto Malaysia lorry

Machine Spec

1 x Stule ST-FC1325L Laser Cut

Dim: 400 x 210 x 170 cm / Wt: 2, 500 kg

1 unit Hwacheon Hi Tech 200B

Dim: 400 x 160 x 175 cm / Wt: 4, 800 kg

1 unit Wele AA1165 CNC

Dim: 320 x 255 x 280 cm / Wt: 7, 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

09/07/25

Time of Operation

09:30 am

Location of Operation

#### 37 Joo Koon Rd

Energy Pipeline Services Pte Ltd

Equipment Use

2 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Hydraulic jack

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated loading area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Shift out machine from production to loading area

- Rigger to insert sling to crane hook

- Secure sling to rigging point of machine

- Lorry crane to hoist up machine and load onto Malaysia lorry

- Release all sling from crane hook

- Secure machine with strapping belt

- Transport and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175