# METHOD OF STATEMENT

Date: 29th Nov 2023

Description of work

To shift out 1 unit machine with accessories from production to loading area for packing, after packing load up onto customer arranged container

Machine Spec

Mazak Intergrex E1060V/8

Refer to attached list

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

04/12/23 – 09/12/23

Location of Operation

#### 51 Corporation Rd

Avitron Pte Ltd

Equipment Use

1 x 100ton mobile crane

Lorry crane

1 x forklift

Scissor lift

Shackle

Wire sling

Webbing sling

Heavy duty rollers

Hydraulic/hand jack

Canvas/metal sheet (for floor protection)

Obstacles

To clear all obstruction in way of crane parking and working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Jack up machine using hydraulic jack

- Insert rollers under machine bed

- Shift out machine from production area to loading bay

- Attached required sling to crane hook

- Secured sling to rigging point of machine to be lifted

- Hoist up machine 1ft above ground to check on balancing

- After well balanced, lift up and load machine onto wooden base for packing

- After packing, load up case onto low-bed

- Release all lifting gears from case

- To secured case with lashing wire and choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175