# METHOD OF STATEMENT

Date: 15th Apr 2019

Description of work

To hoist up 1 unit Chiller from ground floor to building rooftop using mobile crane

Chiller Spec

Dim: 2200 x 1116 x 2060 mm

Wt: 900 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

20/04/2019

Time of Operation

09:00 am

Location of Operation

#### Blk 4 Kaki Bukit Ave 1

Moveon Technologies Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of 50 ton mobile crane at designated location

- Insert sling onto chiller hoisting point

- Hoist up chiller to building roof top

- Lower down chiller onto building roof top

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175