# METHOD OF STATEMENT

Date: 25th Mar 2025

Description of Work

To unload 1 unit machine from 1 x 40ft OT container using customer 20ton overhead crane, uncrate, shift and position to the designated location

Machine Spec

2 unit SC-36 CNC Lathe

Wt: 17, 000 kg

1 unit SC-25 CNC Lathe

Wt: 9, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

1st unit – 28/03/2020

2nd unit – ETA 05/04/2020

3rd unit - TBA

Time of Operation

09:45 am

Location of Operation

#### No 80 Tuas West Drive

Dril-Quip Asia Pte Ltd

Equipment Use

1 x 633 lorry loader

Wire/webbing sling

Shackle

20 ton customer overhead crane

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse 1 x 40ft container into inside factory

- Open up container canvas

- Attached lifting gears onto overhead crane hook and secured to load to be lifted

- Hoist up machine from container and load onto the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Shift and position machine to the designated location

- Unload machine and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175