# METHOD OF STATEMENT

Date: 04th Mar 2020

Description of Work

To unload 1 unit machine from 1 x 20ft OT container, shift and position into factory premise at ground floor

Machine Spec

Ion Nitrding Instrument

Case 1: 244 x 153 x 301 cm / Wt: 2, 030 kg

Case 2: 152 x 217 x 219 cm / Wt: 1, 380 kg

Case 3: 230 x 112 x 87 cm / Wt: 450 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

09/03/2020

Time of Operation

09:45 am

Location of Operation

#### 285 Jalan Ahmad Ibrahim

Panasonic Factory Solutions Asia Pacific

Equipment Use

1 x 633 lorry loader - PALFINGER PK100002F

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the lift

- Open up container canvas using ladder

- Secured sling to machine hoisting point

- Hoist up machine from container and load onto lorry chassis

- Transport lorry to unloading area

- Hoist down case from lorry chassis to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Lower down machine to near ground level

- Insert pallet truck under machine base

- Shift and transport machine into production

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175