# METHOD OF STATEMENT

Date: 26th May 2022

Description of work

To hoist up machine using factory overhead crane, load onto wooden base, shift out to loading bay for packing, after packing, load onto low loader and 1 x 20ft GP container

Machine Spec

3 unit Italcontrol Test machine

Dim: 310 x 275 x 295 cm

Wt: 6, 000 kg each

3 unit base tray

4 unit rack

Materials

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

30-31/05/22

Time of Operation

09:45 am

Location of Operation

#### 2 Gul Circle

Cameron Singapore Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x 10ton forklift

1 x low loader

1 set wire/webbing sling

1 set shackle

Rollers

Pallet truck

Obstacles

To clear all obstruction in way of packing and working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Hoist up machine using factory overhead crane

- Load machine onto wooden base

- Using 10ton forklift to fork out base together with machine to outside factory for packing

- Pack to form a case

- After packing, load up cases onto low loader and container

- Secured case with chocking wood and strapping belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175