# METHOD OF STATEMENT

Date: 11th Mar 2024

Description of work

To hoist up machine and accessories using factory overhead crane, load onto wooden base, shift out to loading bay for packing, after packing, load onto customer arranged 1 x 40ft OT and 4 x 40ft HQ   container

Machine Spec

1 unit HBM 130 Horizontal Boring machine

Packing list refer to attached

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

20-27/03/24

Time of Operation

09:45 am

Location of Operation

#### 10 Gul Circle

OMS Oilfield Services Pte Ltd

Equipment Use

633 lorry loader

Customer 50ton overhead crane

1 set wire

Webbing sling

Shackle

Obstacles

To clear all obstruction in way of packing and working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Using factory 50ton overhead crane to hoist up machine

- Load up machine onto wooden base

- Pack to form a case

- After packing, load up cases onto low container

- Secured case with chocking wood and lashing belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175