# METHOD OF STATEMENT

Date: 29th Sep 2023

Description of work

To unload 3 skid of 1 unit machine from 1 x 40ft HQ container, hoist up to 2nd floor through shutter door opening by using platform and mobile/lorry crane

Machine Spec

MIKROTURN 100

Dim: 310 x 230 x 250 cm

Wt: 6, 728 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

06/10/23

Time of Operation

09:45 am

Location of Operation

#### No 2A Changi North St 2

ETLA Limited

Equipment Use

1 x 70 ton mobile crane

1 x 633 lorry loader

1 x 10ton forklift

1 x platform

1 set rollers

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, mobile/lorry crane operator and forklift driver will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated location for the hoist

- Use forklift to unload skid from container to the ground

- Loosen all nuts attached to machine

- Using forklift to fork up and detach machine from base

- Load up machine onto platform

- Secured machine onto platform with strapping belt

- Mobile crane to hoist up platform to 2nd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean onto the building

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert rollers under machine bed

- Manual pulley is slowly release to control the moving speed of machine

- Shift out machine from platform and position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175