# METHOD OF STATEMENT

Date: 28th Sep 2021

Description of work

To hoist up 2 unit Compressor to 3rd floor rooftop by using mobile crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29/10/21 (Fri)

Time of Operation

09:45 am

Location of Operation

#### JTC Space @ 164 Gul Circle

Beril Engineering (S) Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 633 lorry loader

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile at designated location for the hoist

- Insert sling onto crane hook and secure onto compressor hoisting point

- 50 ton mobile crane to hoist up compressor to 3rd floor rooftop

- Lower down at designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175