# METHOD OF STATEMENT

Date: 03rd Mar 2023

Description of work

1) To shift out and hoist down 1 unit machine from 3rd floor, load up onto lorry chassis and deliver to new factory

2) To shift out machineries from 3rd floor to loading bay via cargo lift, load up onto lorry chassis and deliver to new factory

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

13-15/03/23

(2-3 working days)

Time of Operation

09:30 am

Location of Operation

#### Blk 53 Serangoon North Ave 4 #03-04/04A

Mieux Pte Ltd

Equipment Use

2 x 50ton lorry loader

2-3 x lorry

1 x platform

Roller/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry loader at the designated location for the hoisting work

- Hoist up platform to 3rd floor shutter gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- 50 ton mobile/lorry crane to hoist down machine

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise and deliver to new factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175