# METHOD OF STATEMENT

Date: 07th Dec 2021

Description of work

To hoist up 1 unit machine to 2nd floor through gate opening by using platform and lorry crane

Machine Spec

Ecoca CNC Lathe

Dim: 310 x 162 x 186 cm

Wt: 4, 850 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10-11/12/21

Time of Operation

09:45 am

Location of Operation

#### Blk 65A Jalan Tenteram #02-10

Grindmills Precision Engineering Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x platform

1 set rollers

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated location for the hoist

- Hoist up platform to 2nd floor gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean onto the building

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 1 x lorry loader to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert rollers under machine bed

- Manual pulley is slowly release to control the moving speed of machine when being shift out machine from platform

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175