# METHOD OF STATEMENT

Date: 12th Dec 2018

Description of work

1) To hoist up 3 unit machine to 4th floor by using platform and mobile crane

2) To unload 5 unit machine and accessories at loading bay, shift and position to 4th floor via cargo lift

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/12/2018

Time Of Operation

Est: 11:00 am

Location of Operation

#### 10 Tampines North Drive 4 #04-03

Sintech Automation (S) Pte Ltd

Equipment Use

2 x 50 ton mobile crane

3-4 x lorry

1 x platform

Roller/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- 2 x mobile crane to park at designated location for the hoist

- 1 x mobile crane to hoist up platform to 4th floor door opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 1 x 50 ton mobile crane

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 1 x 50 ton mobile crane to hoist up machine to 5th floor

- Lower down machine onto platform

- Insert rollers/pallet truck onto machine base

- Shift out machine from platform and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175