# METHOD OF STATEMENT

Date: 07th Oct 2019

Description of work

To hoist up 1 unit machine to 2nd floor by using platform and mobile crane

Machine Spec

1 unit Doosan HM8000 CNC

Dim: 650 x 310 x 310 cm

Wt: 20, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10/10/2019

Time Of Operation

Afternoon schedule

Location of Operation

#### 3 Joo Koon Circle

AME International Pte Ltd

Equipment Use

1 x 100 ton mobile crane

1 x 50 ton mobile crane

Lorry crane

1 x low-bed

1 x platform

1 set rollers

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy 2 x mobile crane at designated location for the hoist

- 100 ton mobile crane to hoist up platform to 2nd floor door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean onto the building

- Tied back platform to a competent point and secured using 1 x 50 ton mobile crane

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 100 ton mobile crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert rollers under machine base

- Shift out machine from platform

- Position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175