# METHOD OF STATEMENT

Date: 31st Oct 2023

Description of work

To hoist down 1 unit machine from 3rd floor through shutter door opening by using platform and mobile/lorry crane

Machine Spec

1 unit Mitsui Seiki CNC

Wt: 7, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

03/11/23 (Fri)

Time of Operation

09:45 am

Location of Operation

#### 3 Joo Koon Circle

TQC Precision Engineering Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1-2 x 633 lorry loader

1 x platform

1 set rollers

Pallet truck

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated location for the hoist

- Hoist up platform to 3rd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean onto the building

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production into platform

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- 50 ton mobile crane to hoist up machine from platform

- Swing crane and lower down machine to the ground

- Load machine onto lorry chassis and secured with strapping belt

- Transport and deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175