# METHOD OF STATEMENT

Date: 11th Apr 2019

Description of work

To shift out 1 set of Ishida Weigher machine, including using forklift to assist engineer to dismantle working platform, hoist down from 2nd floor, shift and position into production area at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

20/04/2019

Time Of Operation

09:45 am

Location of Operation

#### 36 Quality Rd

Meiji Seika (S) Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 633 lorry loader

1 x platform

1 x 3 ton battery forklift

Roller/pallet truck

1 set wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the hoist

- Mobile crane to hoist up platform to 2nd floor door opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 1 x 633 lorry loader

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- Hoist down machine from 2nd floor to ground floor

- Insert pallet truck/rollers to machine base

- Shift and transport into production area

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175