# METHOD OF STATEMENT

Date: 24th June 2019

Description of work

To shift out and hoist down 1 unit machine from 2nd floor, load onto Malaysia lorry and deliver to new location

Machine Spec

1 unit VM10Ui CNC

Dim: 175 x 200 x 230 cm

Wt 3, 400 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

27/06/2019   -   09:45 am

Location of Operation

#### 61 Yishun Industrial Pk A #02-01

Automatic Controls & Instrumentation Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x platform

Roller/pallet truck

1 set wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy 2 x lorry crane at designated hoisting area for the lift

- 1 x 50 ton lorry crane to hoist up platform to 2nd floor door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested onto the building opening slab

- Tied back platform to a competent point and secured using 35ton lorry crane

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- 50 ton lorry crane to hoist down machine from 2nd floor to ground floor

- Load machine onto Malaysia lorry

- Secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175