# METHOD OF STATEMENT

Date: 19th June 2019

Description of work

To shift out and hoist down 1 unit machine and 1 unit Bar Feeder from 4th floor, load onto lorry chassis and deliver to new factory

Machine Spec

1 unit Miyano CNC Lathe

Dim: 220 x 150 x 170 cm / Wt 3, 800 kgs

1 unit Bar Feeder

Dim: 335 x 41 x 112 cm / Wt: 300 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/06/2019

Time Of Operation

09:45 am

Location of Operation

#### Blk 20 Woodlands Link #04-29

Fyno Precision Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 633 lorry loader - PALFINGER PK100002E

1 x platform

Roller/pallet truck

1 set wire/webbing sling

1 set shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Mobile crane to hoist up platform to 4th floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested onto the building opening slab

- Tied back platform to a competent point and secured using lorry crane

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- 50 ton mobile crane to hoist down machine

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise and deliver to new factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175