# METHOD OF STATEMENT

Date: 03rd May 2019

Description of work

1) To shift out and hoist down 7 unit machine from 2nd floor to 1st floor, shift and position into premise at #01-04/05

2) To shift out and hoist down 9 unit machine from 3rd floor to 1st floor, shift and position into premise at #01-04/05

3) To shift out 8 unit small machine from 2nd floor to #01-04/05 via cargo lift

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

11/05/2019 (Saturday)

12/05/2019 (Sunday) continue work if unable to finish hoisting on Saturday

Time Of Operation

09:30 am

Location of Operation

#### 3 Star Engineering & Trading

#### Blk 4010 A.M.K Tech Park 1

From #02-05 & #02-04A   -   #01-04/05

From #03-07   -   #01-04/05

Equipment Use

1 x 50 ton mobile

2-3 x lorry crane

1 x platform

1 x platform (for machine pathway)

Roller/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the hoist

- Hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- 50 ton mobile crane to hoist down machine

- Load machine onto below platform

- Shift and position into 1st floor factory premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175