# METHOD OF STATEMENT

Date: 31st Oct 2017

Description of work

1) To shift out and hoist down 3 unit machine from 4th floor, load onto lorry chassis and transport to new factory

2) To shift out 22 unit machine from 4th floor via cargo lift, load onto lorry chassis and transport to new factory

Machine Spec

1 unit TC 731 machine (Dim: 270 x 165 x 230 cm / Wt: 5ton)

1 unit Campro 850 machine (Dim: 220 x 250 x 250cm / Wt: 4 ton)

1 unit Kira KN 40V5 machine (Dim: 380 x 225 x 220 cm / Wt: 8 ton)

Risk Assessment

A copy of Risk Assessment will be attach

Date/Time of Operation

6-7/11/2017   -    09:30 am

Location of Operation

#### 23 Woodlands Terrace #04-00

JHP Technology Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 45 ton lorry crane

3-4 x lorry crane

1 x platform

Roller/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness,  etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the hoist

- Hoist up platform to 4th level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use of strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production and load onto platform

- 50 ton mobile crane to hoist down machine

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise and transport to new factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175