# METHOD OF STATEMENT

Date: 10th Oct 2017

Description of work

To shift out and hoist down 1 unit machine from 2nd floor, load onto lorry chassis and transport to new premise

Machine Spec

1 unit Hardinge 1000 CNC Lathe

Dim: 300 x 220 x 250 cm

Wt: 6,000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date/Time of Operation

12/10/2017   -    09:30 am

Location of Operation

#### 23 Woodlands Terrace #04-00

JHP Technology Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 45 ton lorry crane

1 x lorry crane

1 x platform

Roller/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness,  etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the hoist

- Hoist up platform to 4th level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use of strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production and load onto platform

- 50 ton mobile crane to hoist down machine

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise and transport to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175