# METHOD OF STATEMENT

Date: 01st Nov 2018

Description of work

To shift out and hoist down 1 unit machine from 5th floor, load onto lorry chassis and deliver to new location

Machine Spec

1 unit JMF 12595 Laser Cutting

Dim: 320 x 220 x 210 cm

Wt: 3, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date/Time of Operation

10/11/2018   -    09:30 am

Location of Operation

#### Blk 3015A bi Rd 1 #05-10

Schlegel Elektrokontakt Co (FE) Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 45/50 lorry crane

1 x platform

Roller/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the hoist

- Hoist up platform to 5th floor swing gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- 50 ton mobile crane to hoist down machine

- Load machine onto lorry chassis and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175