# METHOD OF STATEMENT

Date: 02nd July 2019

Description of work

To shift out and hoist down equipment line from 3rd floor, load onto lorry chassis and deliver to scrapyard for disposal

Machine Spec

Reflow Oven line

Dim: 500 x 100 x 160 cm

Wt: 2, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date/Time of Operation

03/07/2019   -    09:45 am

Location of Operation

#### 37 Tuas Ave 8

Mitsui High-Tec (S) Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

Roller/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated location for the hoisting

- Hoist up platform to 3rd floor shutter gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out equipment from production and load onto platform

- Secured equipment onto platform with strapping belt

- Hoist down platform together with machine to the ground

- Load equipment onto lorry chassis and deliver to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175