# METHOD OF STATEMENT

Date: 05th Dec 2023

Description of work

To shift out and hoist down 1 unit machine from 3rd floor by using platform and mobile crane, load onto lorry chassis and deliver to another factory

Machine Spec

1 unit Hexagon 07.10.07 CMM

Dim:

Wt:

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10/01/23

Time of Operation

09:30 am

Location of Operation

#### Windsor Airmotive Asia Pte Ltd

#### Move out: 63 Changi North Crescent

#### Move in: 21 Changi North Crescent

Equipment Use

1 x 50 ton mobile crane

1 x 633 lorry loader

1 x platform

Pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile/lorry crane at designated location for the hoisting

- Hoist up platform to 3rd floor shutter gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production

- Load machine into platform

- Secured machine onto platform with strapping belt

- Hoist down platform together with machine to the ground

- Load onto lorry chassis

- Transport to another factory

- Unload machine from lorry and shift into production at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175