# METHOD OF STATEMENT

Date: 09th Dec 2019

Description of work

To shift out and hoist down machineries from 5th floor through wall opening and load onto Malaysia lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Start: 12/12/2019

End: 31/12/2019

Time of Operation

09:30 am

Location of Operation

#### Blk 155 Kallang Way #05-18/20/26

Tobe Packaging Industries Pte Ltd

Equipment Use

2 x 50 ton mobile crane

1 x 50 lorry crane

3 x 40ft Malaysia lorry

1 x platform

Roller/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy 2 x mobile crane at designated location for the hoisting

- Hoist up platform to 5th floor wall opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to a competent point and secured using 50 ton mobile crane

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- 50 ton mobile crane to hoist down machine

- Load machine onto Malaysia lorry

- Secured machine with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175