# METHOD OF STATEMENT

Date: 11th Sep 2019

Description of work

1) To shift out and hoist down 1 unit Cleaner machine from 3rd floor through swing gate opening by using platform and mobile/lorry crane, load onto lorry chassis and deliver to new factory

2) To shift out 4 unit small machine, 1 unit work bench, 1 unit compressor, 3 unit racking and 2 unit table from 3rd floor to loading bay via cargo lift, load onto lorry chassis and deliver to new factory

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/09/2019 (Saturday)

Time of Operation

09:30 am

Location of Operation

#### Blk 3015A Ubi Rd 1 #03-08

Forward Science Pte Ltd

Equipment Use

1 x 50 ton mobile/lorry crane

2 x lorry crane

1 x platform

Roller/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the hoist

- Hoist up platform to 3rd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested onto the swing gate opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- 50 ton mobile/lorry crane to hoist down machine

- Load machine onto lorry chassis

- Secured machine with strapping belt

- Transport and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175