# METHOD OF STATEMENT

Date: 30th Jan 2020

Description of work

1) To shift out and hoist down 2 unit machine from 6th floor through swing gate opening by using platform and mobile crane, load onto Malaysia lorry

2) To shift out 5 unit small machine and racking from 6th floor to loading bay via cargo lift, load onto Malaysia lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

08/02/2020 (Saturday)

Time of Operation

09:30 am

Location of Operation

#### Blk 3015A Ubi Rd 1 #06-02

Gold Kwang Pte Ltd

Equipment Use

1 x 50 ton mobile

1 x lorry crane

1 x platform

Roller/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the hoist

- Hoist up platform to 6th floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested onto the swing gate opening slab

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from tenant premise and load onto platform

- Secured machine onto platform with strapping belt

- Hoist down platform together with machine to the ground

- Hoist machine from platform and load onto Malaysia lorry

- Secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175