# METHOD OF STATEMENT

Date: 18th Jan 2019

Description of work

To shift out and hoist down 3 unit Bar Feeder from 7th floor, load onto lorry chassis and deliver to new location

Machine Spec

Bar Feeder

Dim: 320 x 220 x 210 cm

Wt: 300 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date/Time of Operation

23/01/2019   -    09:30 am

Location of Operation

#### Blk 20 Woodlands Link #07-39

Singa Precision Engineering

Equipment Use

1 x 50 ton mobile crane

2 x lorry

1 x platform

Roller/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated location for the hoist

- Hoist up platform to 7th floor swing gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out bar feeder from production and load onto platform

- Secured bar feeder onto platform with strapping belt

- Hoist down platform together with bar feeder to ground floor

- Load platform onto lorry chassis and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175