# METHOD OF STATEMENT

Date: 20th Aug 2025

Description of work

1) To hoist up 3 units machine to 3rd floor through shutter door opening by using platform and mobile/lorry crane

2) To hoist down 1 unit machine from 3rd floor through shutter door opening by using platform and mobile/lorry crane

Machine Spec

Hoist up

1 unit Kinwa CL-250 CNC Lathe / Wt: 3, 450 kg

1 unit Kinwa CL-320A/600 CNC Lathe / Wt: 4, 500 kg

1 unit CG Grinder / Wt: 2, 600 kg

Hoist down

1 unit used CNC Lathe / Wt: 3, 400 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

#### JTC Space @ 164 Gul Circle #01-09

Beril Engineering (S) Pte Ltd

Equipment Use

1 x 50 ton mobile crane

3 x 633 lorry loader

1 x platform

Wire sling

Shackle

Rollers/pallet truck

Hand jack

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

PPE Required
Safety helmet

Safety shoe

Safety harness

SelfRetracting lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting location

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Secured machine onto platform with strapping belt

- Riggers to attached sling to crane hook

- Secured sling to rigging point of platform

- Mobile crane to hoist up platform together with machine to 3rd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on building opening slab

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Tied back platform to a competent point and secured using 1 x lorry loader

- Jack up machine and insert rollers under machine bed

- Use manual pulley to slowly released and control moving speed of machine

- Shift out machine from platform

- Position machine at the designated location

- Repeat process till all machine have been hoisted

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175