# METHOD OF STATEMENT

Date: 12th Mar 2025

Description of work

1) To hoist down 1 unit used oven from 4th floor through wall hacking opening by using platform and mobile/lorry crane

2) To hoist up 1 unit new oven to 4th floor through wall hacking opening by using platform and mobile/lorry crane

Spec

Hoist down

1 unit used Reflow Oven

Dim: 470 x 140 x 155 cm

Wt: 1, 800 kg

Hoist up

1 unit new Reflow Oven

Dim: 555 x 145 x 155 cm

Wt: 2, 300 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

05/04/25 (Sat)

Time of Operation

09:45 am

Location of Operation

#### Blk 512 Chai Chee Lane #04-06

Coilcraft Industries (S) Pte Ltd

Equipment Use

1 x 50ton mobile crane

2 x 633 lorry loader

1 x platform

Wire sling

Shackle

Rollers

Pallet truck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

PPE Required
Safety helmet

Safety shoe

Safety harness

Self Retracting lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile/lorry crane at designated location for the lift

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- 1 x mobile crane to hoist up platform to 4 floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on building opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Mobile crane to hoist up oven to 4th floor

- Lower down oven onto platform

- Insert rollers under oven bed

- Use manual pulley to slowly released and control moving speed of oven

- Shift out from platform

- Position into tenant premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175