# METHOD OF STATEMENT

Date: 08th Sep 2025

Description of work

1) To lay out metal plate onto grass patch for transport of mobile crane to hoisting area

2) To unload and hoist down machine & accessories from lorry chassis using lorry crane at loading bay, shift and transport to 5th floor via cargo lift

3) To hoist up 9 units bar feeder to 5th floor through shutter door opening by using platform and mobile crane

Machine Spec

Refer to attached list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17-18/09/25 - lay out metal plate onto grass patch

19-20/09/25 – Unload machine/accessories at unloading area, transport to 5th floor via cargo lift

19-20/09/25 - Hoisting of bar feeder to 5th floor

Time of Operation

Estimate:

11:00 am

Location of Operation

#### 1 Bulim Lane 2 #05-56

Leadtech Precision Engineering

Equipment Use

1 x 50ton mobile crane

2-3 x 633 lorry loader

1 x hoisting platform

Wire sling

Webbing sling

Shackle

Rollers/pallet truck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

PPE Required
Safety helmet

Safety shoe

Safety harness

SelfRetracting lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Signalman

e. Signalman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile/lorry crane at designated location for the lift

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Load up bar feeder onto platform

- Secured bar feeder onto platform with strapping belt

- Mobile crane to hoist up platform together with platform to 5th floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on building opening slab

- Use of strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert roller onto bar feeder stand

- Shift out bar feeder from platform

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175