# METHOD OF STATEMENT

Date: 06th May 24

Description of work

Using customer overhead crane to unload machine from OT container, using rollers to position machine to exact location

Machine Spec

1 unit torque machine

Dim: 670 x 273 x 252 cm /N. Wt: 12.400kgs

Dim: 126 x 63 x 210 cm/N. Wt: 200kgs

Dim: 177 x 125 x 210 cm/N. Wt: 1.500kgs

Dim: 59 x 59 x 88 cm /N. Wt: 150kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

08/05/24

Time of Operation

09:45 am

Location of Operation

#### 10 Gul Circle

OMS Oilfield Services Pte Ltd

Equipment Use

1 x lorry loader

Factory overhead crane

Wire/webbing sling

Shackle

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Deployment of manpower to site

- To reverse container into inside factory

- Open up container canvas

- Rigger to insert sling to overhead crane

- Secured sling to machine rigging point

- Using overhead crane to hoist down machine from container

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and position machine at the designated exact location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175