# METHOD OF STATEMENT

Date : 23rd January 2015

Description of work

To deliver 2 Cases of Equipment (3.3tons). Hoist down from Lorry, Using Lorry Crane. Unloaded, Uncrate. Shift and position at designated location

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

23/01/15

Location of Operation

#### No 11 Calshot Road (SAESPL)

- Equipment Use

1 x Lorry Crane

Wire / Webbing sling

Rollers / Pallet Jack

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who not involved enter unintentionally.

- Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lighting weather.

All debris will clear and dispose

- Lifting Crews

MOM Certified Lifting sup, Rigger man and signalman will be involved in this operation

(attached all Lifting crews certificate)

Safety Aspect

Lifting crew is pre-briefed prior to commencement of job

All personnel at site have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (Safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M cert.

Lifting operation are supervise by MOM certified Lifting Supervisor, Rigger Man And Signal Man.

# JOB SCOPE

Hoist up

- Lorry crane to park at designated area for the lift

- Barricade operation area

- Secured the machine using sling at the correct lifting point

- Hoist up machine just slightly off the ground to check on the center of gravity

- If balancing have been achieved

- Lower down machine slowly for workers to insert rollers/pallet truck

- Shift and position machine to designated area

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Regards: Zailani

Tel:62824175