# METHOD OF STATEMENT

Date: 20th June 2024

Description of work

To unload machine from 1 x 20ft FR container, uncrate, shift and position into factory premise at ground floor

Machine Spec

Dim: 3200 x 4300 x 3200mm /Wt:11.500kgs

Dim: 3010 x 4110 x 3070mm /Wt 9.500kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

25/06/24

Time of Operation

09:45 am

Location of Operation

12 Sixth Lok Yang Rd

Kim Ann Egineering Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Woodblock

Hydraulic/hand jack

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift driver

# Job Scope

# Deploy lorry loader back to back with container

# Using lorry crane to pull out packages from container onto lorry chassis

# Uncrate case and put aside

# Transport and unload machine at door entrance

# Using rollers to transport machine into production area at ground floor

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175