# METHOD OF STATEMENT

Date: 18th Mar 2019

Description of work

To load machine onto 1 x platform and secured with strapping belt, using forklift to transport into inside factory, fork up and level platform with 2nd floor , using pallet to shift out from platform, using ‘A’ frame with 3 ton chain block to lift up to 3rd floor, shift and position to designated location

Date / Time of Operation

28/03/2019   -   09:30 am

Location of Operation

50 Defu Lane 7

Heng Long Leather Co. Pte Ltd

Equipment Use

1 x lorry crane

1 x square platform (230 x 297 cm)

1 set ‘A’ Frame

2 x 3 ton chain block

1 set wire/webbing sling

1 x 10 ton forklift

Pallet truck/rollers

Obstacles

Ensure no obstacles around operation area and route to machine position

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified LG gears forklift operator

Job Scope

# Load machine together with wooden base onto platform

# Secured machine firmly onto platform with strapping belt

# 10 ton forklift to fork up platform together with machine to 2nd floor

# To level platform with 2nd floor flooring

# Using pallet truck to shift out machine from platform

# Shift machine to near ‘A’ frame

# Secure sling onto wooden base and hook onto chain block

# 2 x 3ton chain block to lift up to 3rd floor

# Shift ‘A’ frame away together with base away from hole

# Lower down base to ground level

# Lift up machine and detach from base

# Insert rollers/pallet truck to machine base

# Shift and position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175