# METHOD OF STATEMENT

Date: 29th Aug 2023

Description of work

To unload 5 packages of 1 unit machine from 1 x 40ft HQ container, uncrate, shift and position into factory premise at ground floor

Machine Spec

5 packages of 1 unit Test Bench

Package 1: Wt: 7, 900 kg

Package 2: Wt: 846 kg

Package 3: Wt: 1, 055 kg

Package 4: Wt: 147 kg

Package 5: Wt: 390 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est:

08/09/23

Time of Operation

09:45 am

Location of Operation

12 Tuas Loop

Flowserve Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 10 ton forklift

Low bed

Rollers/pallet truck

Metal sheet/canvas

Woodblock

Hydraulic/hand jack

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift driver

# Job Scope

# Deploy lorry loader back to back with container

# Using lorry crane to pull out packages from container onto lorry chassis

# Using 10 ton forklift to fork down packages to the ground

# Uncrate case and put aside

# Forklift to fork up and detach machine from base

# Transport and unload machine at door entrance

# Using rollers to transport machine into production area at ground floor

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175