# METHOD OF STATEMENT

Date: 29th Dec 2023

Description of work

1) To unload case of 1 unit machine from lorry using forklift, uncrate, detach machine from base, transport to door entrance, shift and position to designated location using rollers

2) To shift out 1 unit CMM from production to loading area, fork up onto lorry chassis for disposal

Machine Spec

Move in

1 unit Explorer Performance 15.15.10 CMM machine

Case Dim: 3228 x 2478 x 2709 mm

Wt: 8, 000 kg

Move out for disposal

1 unit Leader NCL15.15.10 CMM machine

Dim: 320 x 265 x 260 cm

Wt: 8, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

09/01/24

Time of Operation

09:30 am

Location of Operation

23 Loyang Way

GE Aviation

Equipment Use

1 x 633 lorry loader

1 x low loader

1 x 10ton forklift

Hand/hydraulic jack

Woodblock

Rollers/pallet truck

Canvas/metal plate (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift driver

# Job Scope

# Deployment of lorry loader and low loader to site

# Unload forklift from low loader

# Using 10 ton forklift to fork down case from lorry chassis to the ground

# Uncrate case and put aside

# Loosen all nuts form machine bed

# Forklift to fork up and detach machine from base

# Transport and unload machine at door entrance

# Using rollers to transport machine into production area at ground floor

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175